PROPAGATION MODELS IN URBAN AREA FOR WIRELESS COMMUNICATION SYSTEMS IN UHF AND VHF BAND

# Submitted to Prof. M.Irsadi Aksun

Yakup Bayram

Contents:

- Introduction                                                                                               1

2.Models for Macro Cells          						        2

2.1 Empirical Models                        					        3

2.1.1 Rec 529                           						        3

2.1.1.1 VHF Curves                        					        3

2.1.1.2 Okumura- Hata 							        5

2.1.2 Hata Model  								        5

2.1.3 Modified Hata Model							        7

2.1.4 COST 231 Hata Model                                                                      10

2.1.5 Ibrahim and Parsons Model                                                               11

2.1.6 Young’s Propagation Prediction                                                        13

2.1.7 Allsebrook’s Model                                                                            13

2.1.8 McGeehan and Griffiths Model                                                          15

2.1.9 Atefi and Parsons Model              					      15

2.1.10 The Lee Model                                                                                 16

2.2 Theoretical Models                                                                                 17

2.2.1 Flat Edge Model                                                                                17

2.2.2 Walfisch-Bertoni Model                                                                     23

2.2.3 COST 231 Walfisch-Ikegami Model                                                  27

2.2.4 Sakagami-Kuboi Model                                                                     31

2.2.5 MBX Model for Macro Cell    						      32

2.2.6 Ikegami Model                  						      34

3. Models For Micro Cells   							      36

3.1 Haret Model 								      36

3.2 MBX Model For Micro Cells						      40

3.3 Lee’s Model 								      41

3.4 Harley Model 								      45

3.5 Uni-Lund Model 								      46

3.6 COST 231 Walfisch-Ikegami Model 					      47